# AREMA Railway Bridge Strength Check Code Manual

Strength During Deployment, Load to Failure, and Cooper E80 Service Check for Five Deployable Bridges

Key conclusion: E80 Service Check is used for railway service-load verification; Load to Failure is used for generalized vertical-capacity exploration; Strength During Deployment is used for self-weight/deck-weight strength checks during deployment.

# 1. General AREMA Strength-Check Logic

The three AREMA code families for all five bridges ultimately use the same member-strength checking concept. The finite-element solver first obtains the strain of each bar member, then computes stress by σ = Eε and axial force by P = σA. The member is then evaluated using the allowable-stress design logic of AREMA Chapter 15.

Tension members: the allowable tensile force is taken as the smaller value from gross-section yielding and effective-net-area fracture. The current default is Ae = An = 0.9Ag, which preserves the simplified net-area assumption used in the LRFD model.

Compression members: the allowable compressive stress is calculated using the three-region kL/r formulation in AREMA Table 15-1-11. The default effective length factor is k = 3/4, corresponding to the assumed riveted/bolted/welded end-connection condition.

Slenderness: the default limit is L/r <= 100 for primary compression members and L/r <= 200 for tension members. The program uses DCR = max(strength DCR, slenderness DCR).

Loads: unlike the LRFD version, the AREMA mode does not multiply internal forces by 1.5. Instead, it compares service-level member forces with AREMA allowable stresses.

Local plate elements: the b/t and h/t screen for the HSS 8×4×5/16 section is currently retained as a warning. Because this section is a closed HSS shape, it should not be simply treated as an outstanding element that causes automatic failure.

| Check Item | Code Implementation | AREMA Meaning |
| --- | --- | --- |
| Tension gross section | Pallow = 0.55 Fy Ag | Allowable axial tensile stress on the gross section of structural steel. |
| Tension effective net area | Pallow = 0.47 Fu Ae | Effective net-section fracture after accounting for holes and shear lag. |
| Compression, low slenderness | Fa = 0.55 Fy | Allowable compressive stress when kL/r is small. |
| Compression, intermediate slenderness | Fa = 0.60Fy - C(kL/r) | Intermediate-region formula from AREMA Table 15-1-11. |
| Compression, high slenderness | Fa = 0.514π²E/(kL/r)² | Elastic buckling governs the allowable stress. |
| DCR criterion | DCR <= 1.0 | Pass; otherwise the member is marked as not satisfying the criterion in the failure plot. |

# 2. Purpose and Applicable Scenario of the Three AREMA Code Types

| Code Type | Purpose | Load / Analysis Scenario | Applicable Scenario | Main Outputs |
| --- | --- | --- | --- | --- |
| Strength_During_Deploy_AREMA | Checks whether the bridge satisfies AREMA member strength at a selected deployment ratio. | Uses deployment history or geometry update; applies bridge self-weight and deck weight; does not apply the E80 train. | Construction/deployment-stage safety; comparison of stress state and tip deflection at different deployment ratios. | stress plot, failure plot, tip deflection |
| Load_To_Fail_AREMA | After full deployment, progressively increases a generalized vertical load to find the last safe load step under AREMA criteria. | Not a real railway live-load combination; vertical nodal loads are increased until member DCR > 1. | Capacity exploration in the paper, capacity comparison among bridge types, and load/self-weight metric. | maximum safe load, load/self-weight, plots |
| E80_Service_Check_AREMA | Performs AREMA service-load strength checking using the railway Cooper E80 live load. | Moves E80 axle loads and trailing load across the bridge, scans forward and reverse train positions, and includes impact, self-weight, and deck weight. | Determines whether the bridge satisfies AREMA in railway operating service; this should be the primary code for railway bridge design checking. | Pass/Fail, maximum DCR, controlling train position, plots |

Recommendation: if the question is whether the railway bridge satisfies AREMA service checking, use E80_Service_Check_AREMA first. If the question is how much generalized vertical load the bridge can carry, use Load_To_Fail_AREMA. If the question is whether the bridge is safe during deployment, use Strength_During_Deploy_AREMA.

# 3. Railway Load Implementation in the Cooper E80 Service Check

The E80 service check does not apply LRFD load factors. Instead, it builds a railway service live-load model and evaluates the members using AREMA allowable stress. The current implementation is a vertical Cooper E80 moving-live-load strength screen with impact.

E80 axle-load sequence: 40, 80, 80, 80, 80, 52, 52, 52, 52, 40, 80, 80, 80, 80, 52, 52, 52, 52 kip.

Axle-spacing sequence: 8, 5, 5, 5, 9, 5, 6, 5, 8, 8, 5, 5, 5, 9, 5, 6, 5, 5 ft.

Trailing load: 8 kip/ft.

The program moves the train from outside the bridge to outside the bridge, checks both forward and reverse directions, and records the location that produces the maximum DCR.

Impact is calculated automatically as a function of span length using the AREMA Article 1.3.5 formula for rolling equipment without hammer blow.

The current E80 service check does not yet include the Alternate 4-axle load, centrifugal force for curved bridges, wind on loaded/unloaded bridges, longitudinal braking/traction force, continuous welded rail force, or fatigue detail category.